省市体彩中心LNS路由器应急处置预案

（V1.0）

国家体育总局体育彩票管理中心

技术中心

|  | 变更日期 | 变更人 | 变更说明 |
| --- | --- | --- | --- |
| 1 | 2022-07-05 | 王子 | 创建 |

目录

一、适用场景	4

【场景一】：省中心至国家中心数据端专线丢包	4

【场景二】：省中心二次认证服务器不可用	4

【场景三】：省中心LNS路由器出现硬件假活或软件进程卡死	4

【场景四】：SSL网关出现假活导致省中心LNS路由器L2TP进程卡死	4

【场景五】：省中心LNS线路运营商网络出现故障	4

二、预计时间	5

【场景一】	5

【场景二】	5

【场景三】	5

【场景四】	5

【场景五】	5

三、业务影响	5

【场景一】	5

【场景二】	6

【场景三】	6

【场景四】	6

【场景五】	6

四、操作步骤	6

【场景一】	6

【场景二】	7

【场景三】	15

【场景四】	19

【场景五】	24

五、附录	25

一、适用场景

本应急预案主要用于快速恢复解决省市体彩中心LNS路由器或LNS路由器上数据端专线出现故障导致影响终端业务。具体故障场景如下：

【场景一】：省中心至国家中心数据端专线丢包

故障现象：业务监控发现某省VPDN终端出现大量掉线，同时该省中心报障反馈掉线终端出现网络不可连接，运维一线人员登陆内联网接入区路由器上ping对端省中心LNS路由器数据端互联地址，数量100个数据包，当丢包率达到6%以上。

【场景二】：省中心二次认证服务器不可用

故障现象：业务监控发现某省终端在线数远低于日常，同时省中心报障反馈新开机终端或掉线终端无法登录，已在线终端正常，运维一线人员登录省中心LNS路由器上发现认证服务器不可用。

【场景三】：省中心LNS路由器出现硬件假活或软件进程卡死

故障现象：业务监控发现某省VPDN终端出现大量掉线，同时该省中心报障反馈掉线终端出现网络不可连接，运维一线人员登录省中心LNS路由器发现硬件故障或CPU、内存耗尽，需要重启路由器。

【场景四】：SSL网关出现假活导致省中心LNS路由器L2TP进程卡死

故障现象：业务监控发现某省VPDN终端出现大量掉线，同时该省中心报障反馈终端出现SSL网关握手失败，待终端重启后出现网络不可连接，运维一线人员登录省中心LNS路由器发现L2TP进程卡死，需重启路由器。

【场景五】：省中心LNS线路运营商网络出现故障

故障现象：业务监控发现某省VPDN终端出现大量掉线，同时该省中心报障反馈掉线终端出现网络不可连接，报障运营商反馈运营商线路故障。

二、预计时间

【场景一】

执行应急预案的耗时：5-10min；

业务测试及恢复所需的时间：15-30min；

【场景二】

执行应急预案的耗时：5-10min；

业务测试及恢复所需的时间：15-30min；

【场景三】

执行应急预案的耗时：5-10min；

业务测试及恢复所需的时间：15-30min；

【场景四】

执行应急预案的耗时：5-10min；

业务测试及恢复所需的时间：15-30min；

【场景五】

执行应急预案的耗时：无；

业务测试及恢复所需的时间：待运营商反馈；

三、业务影响

【场景一】

处置过程中对现有业务无影响，终端流量会自动切换到另一条线路上。

【场景二】

处置过程对现有业务无影响。

【场景三】

处置过程会导致当前LNS路由器上的业务终端全部掉线，随后掉线的终端会自动切换到另一台LNS路由器上。

【场景四】

处置过程会导致当前LNS路由器上的业务终端全部掉线，随后掉线的终端会自动切换到另一台LNS路由器上。

【场景五】

无处置操作，等待运营商恢复故障。

四、操作步骤

【场景一】

1、故障判断

以江苏为例：

（1）查看PRTG中该专线带宽情况，是否出现带宽饱和。

（2）查看内联网接入区路由器LOG告警信息如下：

NQA entry js js probe fail trap information: pingCtlOwnerIndex: js, pingCtlTestName: js, pingCtlTargetAddressType: 1, pingCtlTargetAddress: 172.7.32.6, pingResultsOperStatus: 1, pingResultsIpTargetAddressType: 0, ping[non auto switch]

（3）ping 江苏LNS2互联地址172.7.32.6，数量100个， 结果显示为“6.00% packet loss”专线丢包。

（4）确认该省剩余链路可承载全省流量，依据PRTG链路监控信息进行评估，在链路切换后确保业务峰值流量不超过链路带宽的95%。

2、处置操作

（1）登陆内联网接入区路由器，手工关闭与省中心LNS端互联的丢包链路。

此操作的影响，需要注意事项：

备注：当此时另一条链路出现中断情况下将影响整个省份售票，需要登录路由器开启因丢包关闭的链路

例：

interface GigabitEthernet0/1/10

shutdown

3、恢复检查

确认路由已经切到另一条链路

在LNS2上show ip route 3.0.0.0 4.0.0.0确认业务路由已指向LNS1，

show结果如下图所示：

R    3.0.0.0/8 [120/6] via 10.100.2.1, 00:00:18, GigabitEthernet0/1

R    4.0.0.0/8 [120/6] via 10.100.2.1, 00:00:18, GigabitEthernet0/1

【场景二】

1、故障判断

（1）登陆LNS路由器端查看二次认证服务器状态，确认所配置的两台二次认证服务器状态是否都已失效。

（2）Ping测试二次认证服务器连通性。

（3）查看reject计数是否持续增长。

思科设备：

show aaa servers ========================================================================== RADIUS: id 1, priority 1, host 10.100.37.2, auth-port 1812, acct-port 1813 State: current DOWN, duration 2639648s, previous duration 0s Dead: total time 0s, count 0 Quarantined: No Authen: request 202634, timeouts 6, failover 0, retransmission 5 Response: accept 202581, reject 47, challenge 0 Response: unexpected 4, server error 0, incorrect 0, time 3ms Transaction: success 202628, failure 1 Throttled: transaction 0, timeout 0, failure 0 Author: request 0, timeouts 0, failover 0, retransmission 0 Response: accept 0, reject 0, challenge 0 Response: unexpected 0, server error 0, incorrect 0, time 0ms Transaction: success 0, failure 0 Throttled: transaction 0, timeout 0, failure 0 Account: request 400838, timeouts 9, failover 0, retransmission 8 Request: start 202434, interim 0, stop 198396 Response: start 202434, interim 0, stop 198395 Response: unexpected 4, server error 0, incorrect 0, time 16ms Transaction: success 400829, failure 1 Throttled: transaction 0, timeout 0, failure 0 Elapsed time since counters last cleared: 4w2d13h14m Estimated Outstanding Access Transactions: 0 Estimated Outstanding Accounting Transactions: 0 Estimated Throttled Access Transactions: 0 Estimated Throttled Accounting Transactions: 0 Maximum Throttled Transactions: access 0, accounting 0 Requests per minute past 24 hours: high - 5 hours, 14 minutes ago: 554 low - 13 hours, 10 minutes ago: 0 average: 14 RADIUS: id 2, priority 2, host 10.100.37.6, auth-port 1812, acct-port 1813 State: current DOWN, duration 2639648s, previous duration 0s Dead: total time 0s, count 0 Quarantined: No Authen: request 2, timeouts 2, failover 1, retransmission 1 Response: accept 0, reject 0, challenge 0 Response: unexpected 2, server error 0, incorrect 0, time 0ms Transaction: success 0, failure 1 Throttled: transaction 0, timeout 0, failure 0 Author: request 0, timeouts 0, failover 0, retransmission 0 Response: accept 0, reject 0, challenge 0 Response: unexpected 0, server error 0, incorrect 0, time 0ms Transaction: success 0, failure 0 Throttled: transaction 0, timeout 0, failure 0 Account: request 4, timeouts 4, failover 1, retransmission 3 Request: start 0, interim 0, stop 1 Response: start 0, interim 0, stop 0 Response: unexpected 4, server error 0, incorrect 0, time 0ms Transaction: success 0, failure 1 Throttled: transaction 0, timeout 0, failure 0 Elapsed time since counters last cleared: 4w2d13h14m Estimated Outstanding Access Transactions: 0 Estimated Outstanding Accounting Transactions: 0 Estimated Throttled Access Transactions: 0 Estimated Throttled Accounting Transactions: 0 Maximum Throttled Transactions: access 0, accounting 0 Requests per minute past 24 hours: high - 13 hours, 13 minutes ago: 0 low - 13 hours, 13 minutes ago: 0 average: 0

华三设备：

display radius scheme ========================================================================== Total 1 RADIUS schemes ------------------------------------------------------------------ RADIUS scheme name: ahtc Index: 0 Primary authentication server: IP : 202.102.192.102 Port: 4645 VPN : Not configured State: Active Test profile: Not configured Primary accounting server: IP : 202.102.192.102 Port: 4646 VPN : Not configured State: Active Second authentication server: IP : 202.102.199.67 Port: 4645 VPN : Not configured State: Active Test profile: Not configured Second accounting server: IP : 202.102.199.67 Port: 4646 VPN : Not configured State: Active Accounting-On function : Enabled extended function : Disabled retransmission times : 50 retransmission interval(seconds) : 3 Timeout Interval(seconds) : 3 Retransmission Times : 5 Retransmission Times for Accounting Update : 5 Server Quiet Period(minutes) : 5 Realtime Accounting Interval(seconds) : 720 NAS IP Address : 60.173.194.1 VPN : Not configured User Name Format : with-domain Data flow unit : Byte Packet unit : One Attribute 15 check-mode : Strict Attribute 25 : Standard Attribute Remanent-Volume unit : Kilo RADIUS server version (vendor ID 2011) : 1.0 Attribute 31 MAC format : HH-HH-HH-HH-HH-HH Attribute 17 carry old password : Disabled ------------------------------------------------------------------ display radius statistics ========================================================================== Auth. Acct. SessCtrl. Request Packet: 70161 3025953 0 Retry Packet: 0 11 - Timeout Packet: 0 11 - Access Challenge: 0 - - Account Start: - 70128 - Account Update: - 2887977 - Account Stop: - 67848 - Terminate Request: - - 0 Set Policy: - - 0 Packet With Response: 70161 3025953 0 Packet Without Response: 0 0 - Access Rejects: 0 - - Dropped Packet: 0 0 0 Check Failures: 0 0 0

华为设备：

display radius-server configuration ========================================================================== RADIUS no response packet count : 10 RADIUS auto recover time(Min) : 3 RADIUS retransmit interval(Sec) : 5 RADIUS authentication source ports : IPv4: 1812 IPv6: 1812 RADIUS accounting source ports : IPv4: 1813 IPv6: 1813 ------------------------------------------------------- Server-group-name : acs Authentication-server: IP:10.100.7.3 Port:1812 Weight[90] [DOWN] Vpn: - Status changing: 2022-07-12 06:55:47 [DOWN] 2022-07-12 06:58:47 [UP] 2022-07-12 06:59:31 [DOWN] 2022-07-12 07:02:31 [UP] 2022-07-12 07:04:01 [DOWN] Authentication-server: IP:10.100.7.5 Port:1812 Weight[1] [UP] [MASTER] Vpn: - Status changing: 2022-07-01 00:24:36 [UP] 2022-07-01 00:24:41 [DOWN] 2022-07-01 00:27:42 [UP] 2022-07-01 00:27:47 [DOWN] 2022-07-01 00:29:37 [UP] Accounting-server : IP:10.100.7.3 Port:1813 Weight[90] [UP] [MASTER] Vpn: - Status changing: 2022-06-30 18:24:24 [UP] 2022-06-30 18:48:15 [DOWN] 2022-06-30 18:51:15 [UP] 2022-06-30 19:08:56 [DOWN] 2022-06-30 19:11:56 [UP] Accounting-server : IP:10.100.7.5 Port:1813 Weight[1] [UP] Vpn: - Status changing: 2022-06-30 18:50:50 [UP] 2022-06-30 19:08:31 [DOWN] 2022-06-30 19:11:31 [UP] 2022-07-01 00:05:39 [DOWN] 2022-07-01 00:08:39 [UP] Protocol-version : radius Shared-secret-key : ****** Retransmission : 5 Timeout-interval(s) : 5 Acct-Start-Packet Resend : NO Acct-Start-Packet Resend-Times : 0 Acct-Stop-Packet Resend : NO Acct-Stop-Packet Resend-Times : 0 ------------------------------------------------------- Total 1,1 printed display aaa statistics authentication ========================================================================== Domain-name : default0 authentication request : 0 authentication response accepts : 0 authentication response rejects : 0 ------------------------------------------------------------------------------ Domain-name : default1 authentication request : 0 authentication response accepts : 0 authentication response rejects : 0 ------------------------------------------------------------------------------ Domain-name : default_admin authentication request : 60942 authentication response accepts : 3931 authentication response rejects : 56252 ------------------------------------------------------------------------------ Domain-name : sctc authentication request : 86380 authentication response accepts : 65642 authentication response rejects : 20738 ------------------------------------------------------------------------------

2、处置操作

（1）与省中心确认是否可以重启二次认证服务器。如反馈不行继续执行下步。

（2）备份配置。

（3）去掉现有二次认证配置。

思科设备：

no aaa authentication ppp default group radius no aaa authorization network default group radius local no aaa accounting delay-start no aaa accounting network default start-stop group radius

华三设备：

domain xxxx undo authentication ppp undo authorization ppp undo accounting ppp

华为设备：

aaa authentication-scheme default undo authentication-mode local radius authorization-scheme default undo authorization-mode local radius commit

（3）将二次认证配置为空，即不做二次认证

思科设备：

aaa authentication ppp default none aaa authorization network default none aaa accounting network default none

华三设备：

domain xxxx authentication ppp none authorization ppp none accounting ppp none

华为设备：

aaa authentication-scheme default authentication-mode none authorization-scheme default authorization-mode none commit

3、恢复检查

查看VPDN连接数是否有增长。

思科设备：

show vpdn session state

华三设备：

dis l2tp session

华为设备：

dis l2tp session lns slot 5

【场景三】

1、故障判断

（1）查看CPU使用情况，状态是否过载到99%~100%：

思科设备：

show processes cpu CPU utilization for five seconds: 3%/0%; one minute: 4%; five minutes: 4% PID Runtime(ms) Invoked uSecs 5Sec 1Min 5Min TTY Process

华三设备：

dis cpu Slot 0 CPU 0 CPU usage: 3% in last 5 seconds 21% in last 1 minute 15% in last 5 minutes Slot 1 CPU 0 CPU usage: 0% in last 5 seconds 0% in last 1 minute 0% in last 5 minutes Slot 3 CPU 0 CPU usage: 4% in last 5 seconds 4% in last 1 minute 4% in last 5 minutes

华为设备：

display cpu-usage Cpu utilization statistics at 2022-07-25 05:30:24 795 ms System cpu use rate is : 3% Cpu utilization for five seconds: 3% ; one minute: 3% ; five minutes: 3%. Max CPU Usage : 59% Max CPU Usage Stat. Time : 2019-07-18 19:18:45 145 ms

（2）查看内存使用情况，状态是否已满：

思科设备：

show processes memory Processor Pool Total: 3946782608 Used: 559422080 Free: 3387360528 lsmpi_io Pool Total: 6295128 Used: 6294296 Free: 832

华三设备：

dis memory Memory statistics are measured in KB: Slot 0: Total Used Free Shared Buffers Cached FreeRatio Mem: 2051924 670404 1381520 0 0 187232 69.3% -/+ Buffers/Cache: 483172 1568752 Swap: 0 0 0 Slot 1: Total Used Free Shared Buffers Cached FreeRatio Mem: 2051924 490788 1561136 0 0 147792 76.2% -/+ Buffers/Cache: 342996 1708928 Swap: 0 0 0 Slot 3: Total Used Free Shared Buffers Cached FreeRatio Mem: 2035320 451708 1583612 0 0 44836 77.8% -/+ Buffers/Cache: 406872 1628448 Swap: 0 0 0

华为设备：

display memory-usage Memory utilization statistics at 2022-07-25 05:30:29 763 ms System Total Memory Is: 8153924 Kbytes Total Memory Used Is: 1472068 Kbytes Memory Using Percentage Is: 18%

（3）查看硬件状态使用情况，正常为Normal：

思科设备：

show environment all Sensor List: Environmental Monitoring Sensor Location State Reading PEM Iout P0 Normal 4 A PEM Vout P0 Normal 12 V DC PEM Vin P0 Normal 218 V AC Temp: PEM In P0 Normal 32 Celsius Temp: PEM Out P0 Normal 34 Celsius Temp: PEM Int P0 Normal 36 Celsius PEM Iout P1 Normal 3 A PEM Vout P1 Normal 12 V DC PEM Vin P1 Normal 218 V AC Temp: PEM In P1 Normal 28 Celsius Temp: PEM Out P1 Normal 32 Celsius Temp: PEM Int P1 Normal 33 Celsius Temp: sTCAM R0 Normal 29 Celsius Temp: Inlet R0 Normal 29 Celsius Temp: Outlet R0 Normal 37 Celsius Temp: QFP Die R0 Normal 37 Celsius Temp: Center R0 Normal 37 Celsius Temp: Oct Die R0 Normal 37 Celsius Temp: CPU Inlt R0 Normal 28 Celsius Temp: CPU VRM R0 Normal 28 Celsius Temp: CPU Die R0 Normal 31 Celsius Temp: FC FANS R0 Fan Speed 60% 22 Celsius Temp: FC FAN0 P0 Fan Speed 60% 22 Celsius Temp: FC FAN1 P1 Fan Speed 60% 22 Celsius

华三设备：

display device verbose Slot No. Board type Status Primary SubSlots --------------------------------------------------------------------- 0 RT-RPE-X5 Normal Master 0 1 RT-RPE-X5 Normal Standby 0 2 N/A Absent N/A N/A 3 RT-FIP-260 Normal N/A 4 Slot 3 : RT-FIP-260(4 Subslots) Subslot 1 : RT-MIC-X-GP8 Subslot 2 : RT-MIC-X-GT8 Subslot 3 : RT-MIC-X-GT8

华为设备：

display device NE20E-S16A's Device status: ------------------------------------------------------------------------------- Slot # Type Online Register Status Role LsId Primary ------------------------------------------------------------------------------- 1 PIC Present Registered Normal OTHER 0 NA 2 PIC Present Registered Normal OTHER 0 NA 3 PIC Present Registered Normal OTHER 0 NA 4 PIC Present Registered Normal OTHER 0 NA 17 NSP Present Registered Normal LC 0 Master 18 NSP Present Registered Normal LC 0 Slave 19 MPU Present Registered Normal MMB 0 Slave 20 MPU Present Registered Normal MMB 0 Master 21 PWR Present Registered Normal OTHER 0 NA 22 PWR Present Registered Normal OTHER 0 NA 23 FAN Present Registered Normal OTHER 0 NA 24 CLK Present Registered Normal OTHER 0 Slave 25 CLK Present Registered Normal OTHER 0 Master

2、处置操作

（1）收取设备诊断信息：

思科设备：

Show tech-support

华三设备：

display diagnostic information

华为设备：

display diagnostic-information

（2）备份配置。

（3）可以软重启则软重启，无法软重启则硬重启，重启前需电话联系故障省份体彩中心技术部人员（见《省市体彩中心技术部人员联系表》）将重启的影响告知，待省中心批复同意后执行。

3、恢复检查

查看VPDN连接数是否有增长。

思科设备：

show vpdn session state

华三设备：

dis l2tp session

华为设备：

dis l2tp session lns slot 5

【场景四】

- 故障判断

（1）查看L2TP Session连接状态是否正常，正常为Established：

思科设备：

show vpdn session L2TP Session Information Total tunnels 113 sessions 1708 LocID RemID TunID Username, Intf/ State Last Chg Uniq ID Vcid, Circuit 52471 11 584 hbtc106..., Vi2.1035 est 03:58:50 167 8348 8 584 hbtc1002..., Vi2.463 est 05:00:58 2359 31308 10 584 hbtc1046..., Vi2.523 est 04:52:13 3023 32282 9 584 hbtc1018..., Vi2.514 est 04:53:35 957 55600 12 584 hbtc1063..., Vi2.778 est 04:25:06 3038 5960 22 584 hbtc1057..., Vi2.705 est 01:44:43 424 58367 17 584 hbtc101..., Vi2.1130 est 03:47:33 2392

华三设备：

display l2tp session LocalSID RemoteSID LocalTID State 22776 11145 1153 Established 63283 11105 1153 Established 38255 11153 1153 Established 13227 11178 1153 Established 53734 11161 1153 Established 28706 11159 1153 Established 3678 11156 1153 Established 44185 11177 1153 Established 19157 11192 1153 Established 59664 11184 1153 Established 50115 11170 1153 Established 25087 11190 1153 Established

华为设备：

Display l2tp session lns slot 0 LocalSID RemoteSID LocalTID RemoteTID UserID UserName ------------------------------------------------------------------------------ 27 63253 5153 8176 11776 01129_170@tc.ln 78 28399 6252 12682 14153 09065_1020@tc.ln 103 11825 5195 55614 30355 16835_1020@tc.ln 108 38019 6925 19273 9355 16903_1020@tc.ln 131 28741 11759 11382 10886 15187_170@tc.ln 138 6 6613 257 30850 21186_170@tc.ln 146 34625 6991 2057 26194 03746_1020@tc.ln 160 27135 6740 7018 9677 11025_1020@tc.ln 184 64433 7053 13193 12870 15772_1020@tc.ln 189 6333 6981 13332 30273 12122_1020@tc.ln 195 16186 5659 14504 8845 07952_1020@tc.ln 199 64287 7053 13193 14532 01268_170@tc.ln 227 20 6613 257 14795 19411_1020@tc.ln 245 10610 6793 1003 14469 09075_1020@tc.ln ------------------------------------------------------------------------------ Total 1750, 46 printed from slot 0

（2）查看L2TP Tunnel建立情况，正常为Established。

思科设备：

show vpdn tunnel L2TP Tunnel Information Total tunnels 113 sessions 1708 LocTunID RemTunID Remote Name State Remote Address Sessn L2TP Class/ Count VPDN Group 584 449 BS484M2 est 59.172.216.90 22 1 842 7547 BS484M2 est 59.172.216.44 9 1 1356 7502 HB-WH-WH-QS-MSE-3.MAN.7750 est 59.172.216.201 0 1 1552 2679 BS100M1 est 59.175.245.134 7 1 1707 1079 HB-WH-WH-ZK-MSE-2.MAN.7750SR12 est 59.172.216.203 0 1 2026 13262 EZ-EC-JB-BAS-1.MAN.ME60 est 58.53.3.74 19 1 2140 9617 EZ-EC-SS-BAS-1.MAN.ME60 est 58.53.2.18 12 1 2484 2590 BS383M1 est 59.175.245.135 4 1 2885 6712 BS484M2 est 59.172.216.64 14 1 3079 1090 BS484M2 est 59.172.216.76 34 1 3503 15819 BS484M2 est 59.172.216.45 19 1

华三设备：

display l2tp tunnel LocalTID RemoteTID State Sessions RemoteAddress RemotePort RemoteName 1153 5999 Established 18 110.19.2.51 1701 Quidway 1207 19527 Established 32 116.113.101.228 1701 nmtc 2346 8563 Established 3 116.112.77.25 1701 nmtc 3478 5741 Established 18 110.19.2.88 1701 Quidway 5480 55199 Established 30 116.113.101.231 1701 nmtc 6423 58860 Established 11 116.114.100.62 1701 NM-XXYP-M60 00-BNG01 6542 15189 Established 5 110.19.4.47 1701 JNZX-HB60-M H01 7616 3606 Established 7 110.19.4.7 1701 JNZX-HB60-M H01 9229 42602 Established 2 110.19.2.9 1701 NMBT-SGXQ-M 6000-BNG02 10515 16266 Established 30 116.116.181.107 1701 nmtc

华为设备：

display l2tp tunnel lns slot 0 LocalTID RemoteTID RemoteAddress Port Sessions RemoteName ------------------------------------------------------------------------------ 1581 9335 221.203.16.88 1701 9 tc.ln 1780 2666 221.202.117.237 1701 10 LW311-BAS-ME60-2 1926 7553 218.25.15.9 1701 30 SY_8841_BAS_ME60TV 2550 129 221.202.137.130 1701 15 tc.ln 3329 4033 218.25.13.10 1701 36 Quidway 3405 10379 123.189.242.235 1701 6 BZ-ME60-01BRAS 3607 59724 218.24.7.121 1701 8 PLD-M6000-03BRAS 4308 2103 218.24.7.216 1701 23 TY-M6000-01BRAS 4322 9388 221.202.137.150 1701 5 Quidway 4454 9193 221.202.4.142 1701 1 Quidway 4521 9637 221.203.16.140 1701 3 tc.ln 4591 1311 221.202.117.219 1701 12 HUAWEI 4860 3602 221.202.117.200 1701 4 HUAWEI 4872 20890 218.24.7.119 1701 23 LS-M6000-02BRAS ------------------------------------------------------------------------------ Total 148, 46 printed from slot 0

2、处置操作

（1）收取设备诊断信息：

思科设备：

Show tech-support

华三设备：

display diagnostic information

华为设备：

display diagnostic-information

（2）备份配置。

（3）如无法show出L2TP Session和Tunnel的结果，则LNS路由器L2TP进程卡死，需重启LNS路由器，可以软重启则软重启，无法软重启则硬重启，重启前需电话联系故障省份体彩中心技术部人员（见《省市体彩中心技术部人员联系表》）将重启的影响告知，待省中心批复同意后执行。

（4）如可show出L2tp Session和Tunnel结果，但SSL网关恢复后L2TP Session仍然没有恢复或者明显增加，执行【场景二】隔离二次认证服务器，如还无法恢复则需软重启LNS路由器。重启前需电话联系故障省份体彩中心技术部人员（见《省市体彩中心技术部人员联系表》）将重启的影响告知，待省中心批复同意后执行。

3、恢复检查

查看VPDN连接数是否有增长。

思科设备：

show vpdn session state

华三设备：

dis l2tp session

华为设备：

dis l2tp session lns slot 5

【场景五】

- 故障判断

- 检查LNS线路联通性：

以安徽为例，在LNS路由器上ping 公网的互联网关地址60.173.194.5，检查联通性。从办公机上ping 60.173.194.5，检查联通性。如出现不通或丢包严重（ping 100个包6%以上丢包率），则为运营商故障。

2、处置操作

电话联系故障省份体彩中心技术部人员（见《省市体彩中心技术部人员联系表》）报障运营商处理。

3、恢复检查

查看VPDN连接数是否有增长。

思科设备：

show vpdn session state

华三设备：

dis l2tp session

华为设备：

dis l2tp session lns slot 5

五、附录

《省市体彩中心技术部人员联系表》

| 省中心 | 联系人 | 职务 | 固定电话 | 手机 |
| --- | --- | --- | --- | --- |
| 北京体彩中心 午休时间：12：00-14：00 客户经理：何磊13581561080 | 潘部长 | 技术部部长 | 010-6319 9708 | 135 2118 9045 |
|  | 李博 | 技术部副部长 | 010-6319 9960 | 139 1006 6821 |
|  |  | 值班电话 | 010-6319 9983 010-6319 9982（机房） |  |
|  |  | 网络值班 | 010-6319 9958 |  |
|  |  | 客服电话 | 010-6319 9666 |  |
| 天津体彩中心 午休时间：12：00-14：00 客户经理：林金凯13718666578 | 姚家圣 | 技术部部长 | 022-2337 9280 | 186 0226 0893 |
|  | 赵晨博 | 职员 |  | 136 7206 0474 |
|  | 韩晓辉 | 职员 |  | 186 0225 0215 |
|  | 马灿 | 职员 |  | 15222671655 |
| 河北体彩中心 午休时间：无 客户经理：邹晓帆13810333136 | 卢强 | 技术部部长 | 0311-8528 6024 | 189 0311 0720 |
|  | 邵优刚 | 技术部副部长 | 0311-6909 2035 | 189 0311 0712 |
|  | 田行懿 | 职员 | 0311-8528 6042 | 135 8211 6900 181 0321 1015 |
|  | 张鹏 | 职员 |  | 135 8235 0551 181 0321 1021 |
|  | 武艳军 | 职员 | 0311-8528 6045 0311-8528 6046 | 181 0321 0995 |
|  | 王鹏远 | 职员 |  | 150 3213 9553 181 0321 0970 |
|  | 韩文浩 | 职员 |  | 189 3116 0659 181 0321 0980 |
|  | 赵玉龙 | 职员 |  | 177 0310 1919 181 0321 1031 |
|  | 王子宇 | 职员 |  | 177 3694 9509 181 0321 0927 |
| 山西体彩中心 午休时间：夏季12：00-15：00 冬季12：00-14：30 客户经理：宋鑫18911865342 | 刘志东 | 技术部部长 | 0351-7620 853 | 186 3451 1814 |
|  | 田剑 | 技术部副部长 | 0351-7620 853 | 186 3451 1852 |
|  | 王军辉 | 职员 | 主：0351-7620 850 备：0351-7620 852 | 186 3451 1851 |
|  | 史琴 | 职员 |  | 186 3451 1849 |
|  | 王振 | 职员 |  | 186 3451 1995 |
| 内蒙古体彩中心 午休时间：12：00-14：30 客户经理：宋鑫18911865342 | 孔帅 | 技术部部长 | 0471-5314 060 0471-5314 080 | 186 8601 7670 |
|  | 张瑞 | 职员 |  | 185 8602 1027 186 8608 2891 |
|  | 张喆 | 职员 |  | 185 8602 1029 186 4724 8633 |
|  | 李玲 | 职员 |  | 185 8602 1030 157 5489 8323 |
| 辽宁体彩中心 午休时间：11：30-13：30 客户经理：邹晓帆13810333136 | 张元勇 | 技术部部长 | 024-2289 5021 | 138 9885 4572 |
|  | 郑博元 | 职员 | 024-2289 0022 |  |
|  | 谭旭 | 职员 |  |  |
|  | 田彦飞 | 职员 |  | 158 4053 0777 |
|  | 董卫东 | 职员 |  |  |
| 吉林体彩中心 午休时间：11：30-14：00 客户经理：邹晓帆13810333136 | 孙晓新 | 技术部部长 | 0431-8781 9177 | 186 8653 2030 |
|  | 朱强 | 技术部副部长 |  | 185 4311 0828 186 8653 2082 |
|  | 陈钢 | 职员 |  | 186 8653 2032 186 8653 2023 |
|  | 肖新东 | 职员 |  | 185 4311 0758 186 8653 2084 |
|  | 宋喆 | 职员 |  | 185 4311 0858 131 9601 9219 |
|  | 李轩 | 职员 |  | 18686532185 |
| 黑龙江体彩中心 午休时间：11：30-14：00 客户经理：邹晓帆13810333136 | 王彦东 | 技术部部长 | 0451-8238 0615 | 138 3616 2651 |
|  | 董忠智 | 职员 |  | 139 3650 6792 |
|  | 宋德纯 | 职员 |  | 139 4608 0753 |
|  | 闫陛行 | 职员 |  |  |
| 上海体彩中心 午休时间：无 客户经理：何磊13581561080 | 刘光伟 | 技术部部长 | 021-6249 0253 | 138 1882 8588 |
|  | 于天石 | 技术部副部长 | 021-6248 8787 021-6248 4121 | 135 0167 6983 |
|  | 喻剑锋 | 职员 |  |  |
|  | 陆胜军 | 职员 |  |  |
|  | 徐海明 | 职员 |  |  |
| 江苏体彩中心 午休时间：12：00-14：00 客户经理：付航13811085228 | 倪海涛 | 技术部部长 | 025-5188 6143 | 189 1598 2661 |
|  | 赵华华 | 职员 | 025-5188 6132 | 189 1598 2710 |
|  | 马琦 | 职员 | 025-5188 6195 | 189 1598 2620 |
|  | 汪志洋 | 职员 | 025-5188 6191 | 133 8209 0796 |
|  | 王建华 | 职员 | 025-5188 6164 | 189 1598 2629 |
|  | 郑芸 | 职员 | 025-5188 6192 |  |
|  | 杨杰 | 职员 | 025-5188 6192 |  |
|  | 郑中 | 职员 | 025-5188 6163 |  |
|  | 张海涛 | 职员 | 025-5188 6192 |  |
|  |  | 值班电话 | 025-5188 6192 |  |
| 浙江体彩中心 午休时间：11：30-13：30 客户经理：何磊13581561080 | 杜春雷 | 技术部部长 | 0571-8509 0590 | 137 7737 0572 |
|  | 周为民 | 职员 | 0571-8509 0593 | 177 6708 1235 |
|  | 沈希杭 | 职员 | 0571-8505 8502 | 139 5815 3776 |
|  | 沈一樑 | 职员 | 0571-8509 0592 | 133 7681 0244 |
|  | 程宇旸 | 职员 | 0571-8505 8506 | 151 5812 9323 |
|  |  | 晚间联系方式 | 0571--8505 8506 |  |
| 安徽体彩中心 午休时间：11：40-14：30 客户经理：谭照庆13910102817 | 魏建军 | 技术部部长 |  | 180 5516 8366 |
|  | 江标 | 技术部副部长 |  | 189 5608 8287 |
|  | 谢晖 | 职员 | 0551-6288 6036-8006 | 189 5608 8269 138 6595 9646 |
|  | 孙洋 | 职员 | 0551-6288 6036-8003 | 189 5608 8232 139 5519 3203 |
|  | 刘斌 | 职员 | 0551-6288 6036-8002 |  |
|  |  | 值 班 | 0551-6288 6036-0 |  |
| 福建体彩中心 午休时间：无 客户经理：邹晓帆13810333136 | 洪小林 | 技术部部长 | 0591-8772 0078 | 133 9500 5552 |
|  | 苏里宁 | 职员 |  | 183 0599 9869 |
|  | 蔡永蒙 | 职员 |  | 137 0509 1355 |
|  | 王喆 | 职员 |  | 135 9905 1450 |
| 江西体彩中心 午休时间：12：00-13：30 客户经理：谭照庆13910102817 | 林春 | 技术部部长 | 0791-8629 4842 | 189 7001 9363 |
|  | 梅帆 | 技术部副部长 |  | 189 7001 9095 |
|  | 曾思琦 | 职员 |  | 189 7001 8961 |
|  | 叶远 | 职员 |  | 189 7097 9829 |
|  | 刘翌 | 职员 |  | 139 7917 9595 |
| 山东体彩中心 午休时间：11：30-13：30 客户经理：谭照庆13910102817 | 张希堂 | 技术部部长 | 0531-8176 3522 | 185 6015 6100 |
|  | 姚洪雷 | 技术部副部长 | 0531-8206 7689 | 185 6015 6017 |
|  | 刘丽宇 | 技术部副部长 | 0531-8205 2818 | 185 6015 6025 |
|  | 许修竹 | 职员 | 0531-6865 5072 | 185 6015 6296 |
|  | 李静竹 | 职员 | 0531-6865 5072 | 185 6015 6818 |
|  | 李家龙 | 职员 | 0531-6865 5072 | 185 6015 6820 |
|  | 刘克猛 | 职员 | 0531-8202 8978 | 185 6015 6060 |
|  | 值班电话 |  | 主：0531-6865 5072 备：0531-8205 2818 0531-6816 5680 |  |
| 河南体彩中心 午休时间：夏季12：00-14：30 冬季12：00-14：00 客户经理：谭照庆13910102817 | 兰振华 | 技术部部长 | 0371-8659 6656 | 18538071516 |
|  | 孔德新 | 职员 | 0371-6356 3601 | 185 3807 1396 |
|  | 刘明洋 | 职员 |  | 136 0867 1355 |
|  | 贺涌 | 职员 |  | 185 3807 1318 |
| 湖北体彩中心 午休时间：无 客户经理：谭照庆13910102817 | 林焰 | 技术部部长 | 027-8732 0425 | 189 8607 8628 |
|  | 张惠杰 | 职员 |  | 180 7103 2631 |
|  | 王津 | 职员 |  | 158 0718 1902 |
|  | 罗鹏 | 职员 |  |  |
|  | 邓文凯 | 职员 |  |  |
|  | 朱筱琳 | 职员 |  |  |
| 湖南体彩中心 午休时间：12：00-14：30 客户经理：谭照庆13910102817 | 施炼 | 技术部部长 | 0731-8517 0396 | 138 7583 7497 |
|  | 张诚 | 职员 | 0731-8517 0398 | 135 4858 3953 |
|  | 陈敬之 | 职员 |  |  |
|  | 黄晓 | 职员 |  |  |
| 广东体彩中心 午休时间：无 客户经理：林金凯13718666578 | 沈翔 | 技术部部长 | 020-3760 3426 | 136 0045 1730 |
|  | 李文灼 | 技术部副部长 | 020-3760 1419 | 189 2222 7862 |
|  | 黎志民 | 职员 |  | 135 0300 5366 |
|  | 刘晓华 | 职员 |  | 135 0000 9104 |
|  | 谢长信 | 职员 |  |  |
|  | 欧尧坤 | 职员 | 020-3760 2594 | 139 2640 8203 |
| 广西体彩中心 午休时间：11：40-14：30 客户经理：何磊13581561080 | 陈宏坚 | 技术部部长 | 0771-5582 005 | 181 7230 0066 |
|  | 廖博艺 | 职员 | 0771-5689 345 传真：0771-5582 010 | 159 7814 0595 |
|  | 余瑜 | 职员 |  | 188 7715 2397 |
|  | 黄亮钦 | 职员 |  |  |
|  | 张强 | 职员 |  | 189 7777 9351 |
| 海南体彩中心 午休时间：12：00-15：00 客户经理：付航13811085228 | 胡鹏 | 技术部部长 | 主：0898-6536 7193 备：0898-6537 1147 | 180 8980 2297 |
|  | 罗坚 | 职员 |  | 189 7618 4401 |
|  | 吴德涛 | 职员 |  | 153 0898 9179 |
|  | 符策富 | 职员 |  | 130 8601 9821 |
|  | 王育居 | 职员 |  | 155 0172 2127 |
|  | 曾春金 | 职员 | 0898-6531 1753 | 138 7631 6822 |
| 重庆体彩中心 午休时间：无 客户经理：宋鑫18911865342 | 王伟 | 技术部部长 | 023-6373 6992 | 180 8302 2449 |
|  | 张雨田 | 职员 |  | 139 0837 0602 |
|  | 周进 | 职员 |  | 189 0838 0322 |
|  | 倪男 | 职员 |  | 181 8305 8033 |
|  | 张启洪 | 职员 |  |  |
|  | 蒲健 | 职员 | 023-6373 6650 | 189 8389 5389 |
|  | 范翔 | 职员 |  | 151 2335 6956 |
| 四川体彩中心 午休时间：无 客户经理：宋鑫18911865342 | 王宏 | 技术部部长 | 028-8554 8255 | 181 0905 0578 |
|  | 张薇 | 职员 |  | 181 0905 0601 |
|  | 滕俊 | 职员 |  | 181 0905 0580 |
|  | 吴斌 | 职员 |  |  |
|  | 周兴刚 | 职员 |  |  |
|  | 黄智敏 | 职员 |  |  |
|  | 李磊 | 职员 | 028-8554 4655 | 181 0905 1301 |
| 贵州体彩中心 午休时间：无 客户经理：林金凯13718666578 | 武翔 | 技术部部长 | 0851-8570 0031 | 139 8411 8845 |
|  | 许翔 | 职员 |  |  |
|  | 熊文 | 职员 |  | 156 8512 5219 |
|  | 杨振宇 | 职员 |  | 185 0855 0314 |
|  | 张权义 | 职员 |  | 176 8508 4810 |
| 云南体彩中心 午休时间：无 客户经理：付航13811085228 | 高建勋 | 技术部部长 | 0871-6462 2520 | 138 8831 1109 |
|  | 张皓 | 职员 | 0871-6459 0177 | 187 8747 4377 |
|  | 倪均 | 职员 |  | 137 0841 3678 |
|  | 刘赛 | 职员 |  | 137 5956 6671 |
|  | 葛晨 | 职员 |  | 183 8716 8592 |
|  | 邹荣禧 | 职员 |  | 137 0840 7305 |
| 西藏体彩中心 午休时间：12：30-15：30 客户经理：何磊13581561080 | 群培 | 技术部部长 | 0891-6388 230 | 189 8999 8655 |
|  | 韩卫 | 职员 |  | 133 2250 5508 |
|  | 肖平 | 职员 |  | 139 8991 7362 |
| 陕西体彩中心 午休时间：夏季12：00-15：00 冬季12：00-14：00 客户经理：付航13811085228 | 郑长江 | 技术部部长 | 主：029-8523 2437 备：029-8526 5853 | 138 0915 6090 133 8928 0080 |
|  | 周振中 | 技术部副部长 |  | 158 2906 6085 |
|  | 毛文 | 职员 |  | 153 5356 5722 |
|  | 霍晔宁 | 职员 |  | 158 0924 0320 |
| 甘肃体彩中心 午休时间：无 客户经理：林金凯13718666578 | 李云鹏 | 技术部部长 |  | 13609377670 |
|  | 刘小龙 | 职员 | 0931-2663 520 | 177 9763 7866 |
|  | 陆洋 | 职员 |  | 136 6935 6565 |
|  | 王谊达 | 职员 |  | 182 9833 2693 |
| 青海体彩中心 午休时间：无 客户经理：宋鑫18911865342 | 杨德全 | 技术部部长 | 0971-8215 736 | 185 9711 7521 |
|  | 韩梅 | 职员 |  | 137 0971 2033 |
|  | 张哲 | 职员 |  | 187 9738 7393 |
| 宁夏体彩中心 午休时间：12：00-14：00 客户经理：林金凯13718666578 | 郑文博 | 技术部部长 | 0951-5018 330 | 189 9511 5200 |
|  | 吕定昌 | 技术部副部长 | 0951-5602 998 | 177 9502 6898 |
|  | 杨永财 | 职员 |  | 137 2337 4110 |
|  | 郑文博 | 职员 |  |  |
|  | 辛明 | 职员 |  |  |
| 新疆体彩中心 午休时间：14：00-16：00 客户经理：付航13811085228 | 刘卫江 | 技术部部长 | 0991-2832 055 | 139 9985 5061 |
|  | 徐小明 | 职员 | 0991-2836 689 | 139 9928 5496 |
|  | 潘璐 | 职员 | 0991-2223 204 | 136 1993 3120 |
|  | 倪阿龙 | 职员 | 0991-2833 033 | 186 9029 9939 |
|  | 值班电话 |  | 0991-2816 263 |  |